| Task | Processor Cycles, % | Code Size, kbytes | Buffer Memory, kbytes |
| --- | --- | --- | --- |
| Service Software: |  |  |  |
| Operating System | 1% | 0.3 | 32.0 |
| 1553 Interface | 1% | 3.0 | 8.0 |
| Command Router | <1% | 0.7 | 10.0 |
| Telemetry Packet Queue | <1% | 0.3 | 10.0 |
| Burst System | 1% | 1.0 | 1500.0 (*) |
| Serial Instrument Interfaces | 1% | 0.5 | 75.0 |
| Housekeeping | <1% | 0.2 | 0.6 |
| Beacon Telemetry | <1% | 0.2 | 0.6 |
| Event Logging | <1% | 0.3 | 0.6 |
| Safing | <1% | 0.5 | 0.3 |
| ### MAG Software | 1% | 0.5 | 1.2 |
| SEP Software | 1% | 1.0 | 2.5 |
| ## SWEA Software: |  |  |  |
| SWEA Moments | 20% | 1.0 | 17.0 |
| SWEA Distributions | 4% | 2.5 | 10.8 |
| SWEA Misc. | 1% | 1.0 | 2.5 |
| ### STE Software | 1% | 1.0 | 3.7 |
| ## PLASTIC Software: |  |  |  |
| PLASTIC Distribution | 6% | 2.0 | 185.6 |
| PLASTIC PHA | 5% | 0.5 | 46.6 |
| PLASTIC Moments | 5% | 1.0 | 45.6 |
| PLASTIC Misc. | 1% | 1.0 | 2.5 |
| # Total | 55% | 18.0 | 1955.1 |
| Available | 100% | 32 (**) | 2048 |
| % Usage | 55% | 56% | 95% |

STEREO IMPACT IDPU Flight Software Resource Requirements Estimates

2001-8-17  D.W.Curtis

(*) Nominal value.  Burst memory will expand to use all unallocated memory space.

(**) 32K is maximum code that can be paged into memory at one time.  Paging code in and out is a complication, but possible.